Dr. D. Y. Patil Institute of Technology, Pimpri, Pune

Smart Electricity Energy Meter

A PBL Project report for

Submitted for the partial fulfillment of the requirements for the

Course: Project Based Learning

(FE, Semester- II)

of

BACHELOR OF ENGINEERING

By

Ms. Anjali Jha        (FN68)

Ms. Irfa Arshad      (FN52)

Ms. Jahnvi Issar     (FN63)

Mr. Krishna Iyer    (FN55)

Mr. Dipak Hole      (FN48)

Mr. Ishan Jain        (FN53)

Under the Guidance of

Mrs.Vishakha Dhanwate Pimpale

Assistant Professor

DEPARTMENT OF FIRST YEAR ENGINEERING

2022-2023

Dr. D. Y. Patil Institute of Technology, Pimpri, Pune

CERTIFICATE

This is to certify that the project work entitled “Smart Electricity Energy Meter” is a bonafide work carried out by Ms. Anjali Jha (FN68) in partial fulfillment of the requirements for the subject Project Based Learning (FE, 2nd Semester) of degree of Bachelor of Engineering in First Year Engineering from Dr. D. Y. Patil Institute of Technology, Pimpri during the academic year 2022-2023.

Project Guide                              Dr. R. G. Biradar                                          Dr. P.S. Warke

Dean F.E.                                                      Dean F.E.

Acknowledgement

It gives us a great sense of pleasure to present the report of the Project Work undertaken during B. Tech. First Year. We owe a special debt of gratitude to our Project Coordinator Dr. Rujuta Joshi, Department of Electronics And Telecommunication, Dr. DY Patil Institute Of Technology, Pune for her constant support and guidance throughout the course of our work. It is only her cognizant efforts that our endeavors have seen light of the day.

Our deepest thanks to Project Guide Mrs. Vishakha Dhanwate Pimpale  and Mrs. Rupali Deshmukh faculty of Electronics Department, the Guide of the project for guiding and correcting various documents of ours with attention and care. A special thanks to our team members for constant support, co-operation and for being efficient throughout the journey of making this project.

Irfa Arshad (FN52)

Anjali Jha (FN68)

Jahnvi Issar(FN63)

Dipak Hole(FN48)

Krishna Iyer (FN55)

Ishan Jain (FN53

Smart Electricity Energy Meter

Our project is about IOT based electricity energy meter using ESP32 and monitor data on Blynk application. With the current technology, we need to have a check on the meter reading in the meter reading room. Thus, monitoring and keeping track records of our electricity consumption is a tedious task. To automate this, we can use Internet of Things. The components used in this project are ESP32 Wi-Fi Module, ZMPT101B AC Voltage Sensor Module, SCT-013-030 Non-invasive AC Current sensor, Potentiometer 10K, resistor 10K, Resistor 100-ohm, Capacitor 10 microfarad, Breadboard.

| Sr. No. | Roll No. | Name of the student | Signature |
| --- | --- | --- | --- |
| 1 | FN68 | Anjali Jha |  |
| 2 | FN63 | Jahnvi Issar |  |
| 3 | FN52 | Irfa Arshad |  |
| 4 | FN55 | Krishna Iyer |  |
| 5 | FN48 | Dipak Hole |  |
| 6 | FN53 | Ishan Jain |  |

Name and signature of mentor:

CONTENTS

Electricity is an essential part of modern life. People use electricity for lighting, heating, cooling, and refrigeration and for operating various appliances like computers, electronics, machinery, and public transportation systems. Nearly 0.07% of Indian households (0.2 million) have no access to electricity. The International Energy Agency estimates India will add between 600 GW to 1,200 GW of additional new power generation capacity before 2050.

To overcome the rapid consumption of electricity we have introduced smart electricity meter. A smart meter is an electronic device that records information such as consumption of electric energy, voltage levels, current, and power factor and communicates the information to the consumer and electricity suppliers.

Smart meters provide a way of measuring electricity consumption in near real-time. This allows utility companies to charge different prices for consumption according to the time of day and the season. Smart metering offers potential benefits to customers. These include

- an end to estimated bills, which are a major source of complaints for many customers

- a tool to help consumers better manage their energy purchases—smart meters with a display outside their homes could provide up-to-date information on electricity consumption and thus help people to manage their energy use and reduce their energy bills.

As a result, smart meters are considered an essential technology to the decarbonisation of the energy system.

1.INTRODUCTION

At present, Electricity is the essential commodity in the world for human life today. Every home, offices, companies, industries require electricity connection for their functioning. Due to rapid increase in human population and the human ‘s dependency towards electrical energy, the demand for electricity has increased many folds, causing deficit of electrical energy during peak hours. In order to cope up with the energy challenges, it is necessary to modernize the electrical system. Internet of Things (IoT) technology can be employed to energy consumption and distribution in different scenarios. Latest development in IoT and digital technology, the concept of smart city is becoming smarter compared to earlier years. Therefore, it is necessary to switch over to innovative and better alternatives such as smart grid, smart metering and zero energy building that will assist to minimize reliance on these assets by minimizing energy consumption and improving usage of renewable energies. This will in turn increase the efficiency of power and energy manages system. Accurate metering, detection of theft and implementation of proper tariff and billing system would manage the consumption of electrical energy. Collecting meter reading is one of the most difficult procedures in billing. The traditional electrical energy meter data collection is such that a person from the utility provider visits the consumer sites periodically to note the meter reading. This procedure has lot of drawbacks such as, it is time consuming, tiresome, requires more human resource, human error and even corruption is probable. The process may be interrupted due to bad weather conditions, also if the consumer is not available, the billing will be pending and human operator needs to revisit. India is facing energy deficit during peak hours. Low voltage during peak hours has been reported as a major power quality issue. Load shedding is a common power management practice followed by the utility providers. Energy conservation has great significance in this scenario of increasing electrical energy demand.The present system of energy billing is error prone, time consuming and laborious. Errors get introduced at every stage of energy billing like errors with electro-mechanical meters, human errors while noting down the meter reading. These errors can be overcome by using the smart energy meter. The main objective of the proposed work is to develop a smart energy meter. The ESP32 module takes the pulse from the energy meter, calculate units and displays the reading on the LCD. The reading is stored in built in EEPROM so, in case of power failures it continues the computation. The reading of the energy meter is also sent to the cell phone of the user in the form of a message through Wi-Fi modem. The proto type system is powered by external power supply that takes the ac power and converts it into dc power and is fed to Arduino and Wi Fi.

2. Literature Survey

The advancement of the Internet of Things has been emerging day by day. The Internet of Things (IoT) revolves between M2M that is embedded with electronics, software, sensors, actuators that assist users in monitoring and controlling devices remotely and efficiently [1]. In the IOT based system object and living being are provided with unique identifiers with the ability to transfer data [2]. The area of IOT has amplified from the convergence of wireless technologies, microelectromechanical systems and the Internet [2]. Nowadays IOT technology is being applied in many areas like electricity, gas, water etc. to make our life automated. Nowadays due to the excessive use of the internet, these areas become computerized and an online payment system makes possible. But accessing meter reading is a manual process and has the possibility of error which causes high revenue cost [3]. Automatic Meter Reading (ARM) technology facilitates the assessment of energy consumption and analysis of data for billing and payment. ARM technology requires bringing the device online and connecting the device with the internet which is in other terms Internet of Things [1]. ARM technology using wireless communication is cheaper than wired medium. Hence, Wi-Fi is more suitable for the proposed system as it is very common in every residence. According to statistics, the number of smart phone users in the world was 2.87 billion in 2017 [4]. These statistics indicates usage of Android phones in the World. Hence, we choose the Android platform to implement our system. People have to check their electricity meter reading manually. In India, there are some people who don’t understand meter reading. Hence, they do not know the used amount of electricity. The aim of the work is to design and develop a real-time low- cost energy meter monitoring systems integrated with cloud services along with an android application. In [7] the author proposed a Wi-Fi based single-phase smart meter based on IoT. The author used a digital meter, ESP8266 Wi-Fi module and a web application for the user interface. The ESP8266 Wi-Fi module has attached into the meter. The ESP8266 Wi-Fi module has been implemented by TCP/IP protocol as the means of communication between the meter and web application. The proposed system is secured and open source but costly. The system consists of the electricity meter which measures the electricity bill and informs the consumer about the number of units consumed and associated costs with it. The microcontroller coordinates the whole system with the help of its different components connected to it. In [8] an Automatic Meter Reading (ARM) based Power Meter with Wi-Fi Communication Module scheme has been proposed and developed software based on Linux. In [9] the author presented a survey report on the utilization of smart electricity meters and some key aspects of the metering process. As well as opportunities arising due to the advent of big data and the increasing popularity of the cloud environments challenges are highlighted by the author. In [10] the author proposed a system where Arduino Uno has been developed with an Ethernet shield that can monitor all the necessary activities in the flow of electricity, the use of current and electricity costs and mentioned hope of reducing the problems associated with payment, calculate the cost of the unit of electricity. In [11] the idea of smart energy meter using IOT and Arduino have been introduced. Energy meters which is already installed at houses are not replaced, but a small modification on the already installed meters can change the existing meters into smart meters. Users will get a notification regarding energy consumption. In [3] the author proposed a real-time monitoring system for residential energy meter. The presented system provided inclusive and continuous access to energy consumption to the consumer by exploiting the advancement of  IOT technology

3. Problem Defination

A smart electricity energy meter is an electrical device that measures the electrical power being consumed and this allows the energy consumed over time (by a residential building, a business, or an electrically powered device) to be determined.

It is used to operate continuously measuring the instantaneous voltage (volts) and current (amperes) to give energy used (in joules, kilowatt-hours etc.). Meters for smaller services (such as small residential customers) can be connected directly in-line between source and customer.

4.System Design

We have created an IoT-based Smart Electricity Energy Meter using ESP32 and the updated Blynk 2.0 application. By using the best current sensor (SCT-013) and voltage sensor (ZMPT101B), we can measure voltage, current, power, and total energy consumed in kWh. The readings will be sent to the Blynk 2.0 application and displayed on a dashboard accessible from any location. In case of power outages, the energy meter data will be stored in ESP32’s EEPROM, ensuring continuous readings. Let’s start our project to automate electricity consumption monitoring.

4.1 Components

| Sr. No. | Components | Quantity |
| --- | --- | --- |
| 1 | ESP32 WiFi Module | 1 |
| 2 | SCT-013-030 Non-invasive AC Current Sensor | 1 |
| 3 | ZMPT101B AC Voltage Sensor Module | 1 |
| 4 | 16*2 LCD Display | 1 |
| 5 | Potentiometer 10k | 1 |
| 6 | Resistor 10K | 2 |
| 7 | Resistor 100 ohm | 1 |
| 8 | Capacitor 10 microF | 1 |
| 9 | Connecting Wires | 10 |
| 10 | Breadboard | 1 |

5. Implementation

The connection diagram is simple. Both the Sensor, i.e. SCT-013 Current Sensor & ZMPT101B Voltage Sensor VCC is connected to Vin of ESP32 which is a 5V Supply. The GND pin of both the modules is connected to the GND of ESP32. The output analog pin of the ZMPT101B Voltage Sensor is connected to GPIO35 of ESP32. Similarly, the output analog pin of SCT-013 Current Sensor is connected to GPIO34 of ESP32. You need a two resistor of 10K & a single resistor of 100 ohms connected along with a 10uF Capacitor.

Apart from the circuit part, the AC wires where the current and voltage needs to measured are connected to the input AC Terminal of Voltage Sensor. Similarly, the current sensor clip doesn’t have any connection and a single live wire or neutral wire is inserted inside the clip part as shown in the above circuit.The 16×2 LCD used in this project is optional. There is no need to connect the LCD as we will be monitoring the ESP32/SCT-013 ZMPT101B/ Energy Meter Data on Blynk Application. In case you want to connect the LCD, you need so many connections. Connect the pin numbers 4, 6, 11, 12, 13, 14 of LCD to ESP32 D13, D12, D14, D27, D26, D25 pins. Also connect the LCD 1, 5, 16 Pin to GND & 2, 15 Pin to 5V VCC. Use a 10K Potentiometer at Pin 3 of LCD to adjust the LCD Contrast.

5.1 Setting Up Blynk Application

Blynk is an application that runs over Android and IOS devices to control any IoT based application using Smartphones. It allows you to create your Graphical user interface for IoT application. Here we will display the IoT Energy Meter Data on Blynk Application.

So download and install the Blynk Application from Google Play Store. IOS users can download from the App Store. Once the installation is completed, open the app & sign-up using your Email id and Password.

From the dashboard create a new project and select ESP32 & Wifi Connection.

Then drag & drop or add 4 widgets and assign the variable as per code and then email the authentication code.

You will get the authentication code in the mail. Copy this authentication code. This will be used in your code.

5.2 Source Code

#include <LiquidCrystal.h>

LiquidCrystal lcd(13, 12, 14, 27, 26, 25);

#define BLYNK_PRINT Serial

#include "EmonLib.h"

#include <WiFi.h>

#include <WiFiClient.h>

#include <BlynkSimpleEsp32.h>

EnergyMonitor emon;

#define vCalibration 83.3

#define currCalibration 0.50

BlynkTimer timer;

char auth[] = "hsYG_5da4gdP9jZkL18O5RNcJSrBT-Ou";

char ssid[] = "Alexahome";

char pass[] = "loranthus";

float kWh = 0;

unsigned long lastmillis = millis();

void myTimerEvent()

{

emon.calcVI(20, 2000);

kWh = kWh + emon.apparentPower * (millis() - lastmillis) / 3600000000.0;

yield();

Serial.print("Vrms: ");

Serial.print(emon.Vrms, 2);

Serial.print("V");

Serial.print("\tIrms: ");

Serial.print(emon.Irms, 4);

Serial.print("A");

Serial.print("\tPower: ");

Serial.print(emon.apparentPower, 4);

Serial.print("W");

Serial.print("\tkWh: ");

Serial.print(kWh, 5);

Serial.println("kWh");

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Vrms:");

lcd.print(emon.Vrms, 2);

lcd.print("V");

lcd.setCursor(0, 1);

lcd.print("Irms:");

lcd.print(emon.Irms, 4);

lcd.print("A");

delay(2500);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Power:");

lcd.print(emon.apparentPower, 4);

lcd.print("W");

lcd.setCursor(0, 1);

lcd.print("kWh:");

lcd.print(kWh, 4);

lcd.print("W");

delay(2500);

lastmillis = millis();

Blynk.virtualWrite(V0, emon.Vrms);

Blynk.virtualWrite(V1, emon.Irms);

Blynk.virtualWrite(V2, emon.apparentPower);

Blynk.virtualWrite(V3, kWh);

}

void setup()

{

Serial.begin(9600);

Blynk.begin(auth, ssid, pass);

lcd.begin(16, 2);

emon.voltage(35, vCalibration, 1.7); // Voltage: input pin, calibration, phase_shift

emon.current(34, currCalibration); // Current: input pin, calibration.

timer.setInterval(5000L, myTimerEvent);

lcd.setCursor(3, 0);

lcd.print("IoT Energy");

lcd.setCursor(5, 1);

lcd.print("Meter");

delay(3000);

lcd.clear();

}

void loop()

{

Blynk.run();

timer.run();

}

5.3 Testing: IoT Based Electricity Energy Meter using ESP32 & Blynk

The ESP32 Board will try connecting to the wifi Network using the given SSID & Password. The LCD Display will light up with the message Initially.When no load is connected or when the load is powered off the Current and Voltage parameters should be almost 0. If it shows some other value, then you need to modify the following calibration factor in the code.Now when the load is connected, the LCD Display will display the Voltage and Current value on LCD Screen along with Power Consumption and total kWh units.The energy meter data is uploaded to Blynk Application after the interval of every 5 seconds. The data can be observed on Serial Monitor as well as Blynk Application.

6.Conclusion and Future scope

In the aera of smart city, smart grid advancement, prototype smart energy meter is a step forward and it mainly focusses on the connectivity & networking factor of the IoT. In this system, an energy consumption calculation based on the counting of calibration pulses is designed and implemented using Arduino Uno MCU in embedded system domain. In the proposed work, IoT based meter reading system is used to continuously monitor the meter reading, current energy consumption, theft detection and service provider can disconnect the power source whenever the consumer does not pay the monthly bill. All these information is sent in the form of message alerts to the consumer mobile phone. Also, it eliminates the human intervention, delivers effective meter reading, prevent the billing mistakes.

Smart Energy Metering using IoT is a revolutionary technology that has the potential to transform the way we consume & track energy. By providing real-time monitoring and control of energy consumption, it can help users to reduce their energy usage, save money, and promote sustainable energy consumption

In future, the proposed system can be extended to be used as prepaid energy meters. These meters can be recharged according to the user’s need thus saving any additional cost in the billing.

Within this armoury of technology at our disposal, smart meters will stand as an indispensable tool in the development of a more sustainable future. According to Mordor Intelligence, the global total shipment of smart meters is estimated to reach 198.53 million units by 2026.

References

[1].Nabil Mohammad, AnomadarshiBarua and Muhammad Abdullah Arafat† Department of Electrical and Electronic Engineering, ―A Smart Prepaid Energy Metering System to Control Electricity Theftǁ, International Conference on Power, Energy and Control (ICPEC) 2013.

[2]. Visalatchi S and Kamal Sandeep K, ―Smart Energy Metering and Power Theft Control using Arduino& GSMǁ, 2nd International Conference for Convergence in Technology 2017.

[3]. Mohammed HosseinandHosseinHejazi Design and Implimentation of an Internet of Things Based Smart Energy Metering 6th IEEE International Conference on Smart Energy Grid Engineering in 2018.

[4]. BibekKanti Barman, Shiv NathYadav, Shivam Kumar and SadhanGope, ―IOT Based Smart Energy Meter for Efficient Energy Utilization in Smart Gridǁ, IEEE conference 2018.

[5]. Garrab, A.; Bouallegue, A.; Ben Abdallah, ―A new AMR approach for energy saving in Smart Grids using Smart Meter and partial Power Line Communicationǁ, IEEE First International Conference on Renewable Energies and Vehicular Technology (REVET), pp. 263 – 269, march 2012.

[6]. SomashekharSwamy and P K Kulkarni Realization of Energy Management System, Impendents on its path in India. International Journal of Advanced Research in Electrical,Electronics and Instrumentation Engineering Volume 9, Issue 5,May 2020.

[7]. Smart Power Meter for the IoT, M. Carratu; M. Ferro; A. Pietrosanto; V. Paciello, 2018.

[8]. Energy theft and defective meters detection in AMI using linear regression, SookChin Yip; Chia-Kwang Tan; Wooi-Nee Tan; Ming-Tao Gan;Ab-Halim Abu Bakar, IEEE 2017.

[9]. Apoorva ,Ashwini and Nisarga B.E project report on IoT based smart energy meter with power theft information and automatic billing Global Academy of Technology Bangaluru for the year 2019-20.

[10]. Eyad I. Abbas, Mohammed E. Safi and Mustafa Abd-alreesJaber Department of Electrical Engineering, ―Design and Implementation Prepaid Energy Meter Supported by RFID and GSM Technologiesǁ, International Conference on Advanced Science and Engineering (ICOASE), Kurdistan Region, Iraq 2018